1. IDENTIFICATION OF THE SUBSTANCE PREPARATION AND OF THE COMPANY

■ Product name : TOWREX AS 603 ■ Manufacturer : DONGBO S. C. CORPORATION 162-4, Sinduck-ri, Sungnam-Myoun Cheonan-si, Choongcheongnam-do, Korea Tel: +82-41-55 3-0883 / Fax: +82-41-553-0803 ■ Intended use : Plastic additive (Antistatic)

2. COMPOSITION / INFORMATION ON INGREDIENTS

Chemical Name; Quantity; CAS NO.. Polyoxy ethylene stearyl amine; 26635-92-7. Fatty amine ester; 52497-24-2. Polyol ester; 31566-31-1.

3. PHYSIOLOGICAL AND HEALTH HAZARD DATA

---------------- Emergency Overview----------------------------------  Major health hazard : N.A(No target organs). ---------------- Potential health effects ------------------------------: - Eyes : May cause slight irritation. - Skin : May cause slight irritation. - Inhalation : May cause headache, frustration. - Ingestion : Throw up, stomachache, nausea  Possibility of causing cancer : N.A

4. FIRST AID MEASURES

■ Inhalation : Remove from exposure immediately. If necessary, artificial respiration(rescue breathing) is needed. Get medical attention. ■ Skin : Remove everything you were wearing such as contaminated clothing , shoes, accessories, then should wash them(about 15 – 20 min)with detergent and large amounts of water.. ■ Eye : Immediately wash eyes with large amount of water or normal saline, occasionally lifting upper and lower lids, until no evidence of chemical remains, Get medical attention. ■ Ingestion : If vomiting occurs, lower your head than hips to help prevent aspiration. Contact a physician.

5. FREE-FIGHTING MEASURES

■ Extinguish media : Water, carbon dioxide, foam or dry powder  Regulation of ways to extinguish : N.A. ■ Fire and explosion hazards - Flash point : > 200℃ - Auto ignition temperature : N.A. ■ Special fire fighting procedures : - Move the container from fire area if there is no danger. - Use water spray to cool fire exposed containers. - Reduce smoke using water spray - Avoid breathing smoke, fumes and decomposition products. ■ Hazardous Result after fire : N.A

MATERIAL SAFETY DATA SHEET

6. SPILL OR LEAK PROCEDURE

■ Steps to be taken in case of release or spill : - Transfer spilled material to secure containers. Then, put them away from water facility. - For health : Isolate hazard area, then shouldn’t take in dust and smoke using mask or something - For environment : Collect spilled materials in appropriate container for disposal - All materials collected should be thrown away according to the local solid waste disposal regulation.

7. HANDING AND STORAGE

■ Storage & Handling - Store and handle in accordance with all current regulations and standards. - Store cool place that air circulates well. Avoid physical problem.

8. SPECIAL PROTECTION INFORMATION

 Exposure Limits :  Install the fan to maintain fresh air  Eye protection : Wear splash resistant safety goggle. Provide and emergency eye wash fountain and quick drench shower in the immediate work area  Wear adequate clothing to minimize physical contact  Use chemical resistant gloves  Other instructions : N.A  Under conditions of frequent use or heavy exposure, respiratory protection may be needed. Any chemical cartridge respirator with organic vapor cartridge(s). Any chemical cartridge respirator with a full facepiece and organic vapor cartridge(s) Any air-purifying respirator with a full facepiece and an organic vapor canister. For unknown concentrations or immediately dangerous to life or health – Any supplied-air respirator with full facepiece and operated in a pressure-demand or other positive-pressure mode in combination with a separate escape supply. Any self-contained breathing apparatus with a full facepiece

9. PHYSICAL DATA

■ Appearance : Yellow pellet  Odor : slight smell  PH : 6 – 8(1% - sol,n) ■ Melting point : 46 ℃  Boiling point : N.A ■ Vapor Pressure : N.A  Vapor density : N.A  Specific gravity : 0.865(80℃)  Viscosity : 28cps(80℃)  Molecular weight : ~ 328~ ■ Explosion : N.A ■ Solubility : It’s mixed with Aromatic solvent, Chloroform

10. STABILITY AND REACTIVITY

 Stability : Stable at high temperature and pressure ■ Conditions to avoid : Avoid heat, flames, sparks and other sources of ignition. ■ Incompatibility (materials to be avoided) : strong base, acid ■ Hazardous decomposition products : N.A ■ Possibility of Polymerization : N.A

11. TOXICOLOGICAL INFORMATION

■ Toxicity data : N.A  Acute toxicity level : Not enough information  Possibility of causing cancer : N.A

12. ENVIROMENTAL HAZARDS

 Information of environmental poison : No  Possibility to be accumulated in body : No  Is this poisonous to ecosystem / aquatic? : No  Poisonous to soil : No

13. DISPOSAL CONSIDERATION

■ Waste disposal : All materials should be packaged, labeled, transported according to the local solid waste disposal regulation.

14. TRANSPORTATION INFORMATION

 Classification is not assigned

15. REGULATORY INFORMATION

 Industry safety law : No regulation  Toxic chemical material management law : No regulation  The fire service act : No regulation

16. OTHER INFORMATION

This Information is from MSDS of Korea Occupational Safety & Healthy Agency  Date prepared : 1996. 5. 2.  Revision date : 6 / 2002. 1. 10.  Approved by : T. H. HAM / Environmental Manager The information contained herein is provided for the purpose of disclosing product hazards and does not constitute product specifications. All warranties, either expressed or implied, are hereby disclaimed.