1. IDENTIFICATION OF THE SUBSTANCE PREPARATION AND OF THE COMPANY

■ Product name : TOWREX AS 700S ■ Manufacturer : DONGBO S. C. CORPORATION 162-4, Sinduck-ri, Sungnam-Myoun Cheonan-si, Choongcheongnam-do, Korea Tel: +82-41-55 3-0883 / Fax: +82-41-553-0803 ■ Intended use : Plastic additive (Antistatic agent)

2. COMPOSITION / INFORMATION ON INGREDIENTS

Chemical Name; Quantity; CAS NO.. Polyoxy ethylene stearyl amine; 26635-92-7. Silica.

3. HAZARDS IDENTIFICATION

NFPA( level : 0 – 4) : Health Rating=3, Flammability Rating=1, Reactivity Rating=0 ---------------- Emergency Overview----------------------------------  This could irritate respiratory organ, skin and eyes. Do not take mist in. Put it away from things that could cause fire. Containers should be closed perfectly. After using it, you must wash hands. When using it, it’s needed to refresh air continuously. ---------------- Potential health effects ------------------------------:  Eye contact: May cause irritation.  Skin contact: May cause irritation.  Inhalation : May cause headache, frustration.  Ingestion : Throw up, stomachache, nausea.

4. FIRST AID MEASURES

■ Inhalation : Remove from exposure immediately. Be medically aided if symptoms such as cough become worse.. ■ Skin : Remove everything you were wearing such as contaminated clothing , shoes, accessories, then should wash them(about 15 – 20 min)with detergent and large amounts of water. Contact a physician if irritation continues ■ Eye : Immediately wash eyes with large amount of water or normal saline for 15 minutes, occasionally lifting upper and lower lids, until no evidence of chemical remains, Get medical attention. ■ Ingestion : Induce to throw up and contact a physician.

5. FREE-FIGHTING MEASURES

■ Extinguish media : Regular dry chemical, water and foam.  Fire And explosion hazards - Flash point : >200℃(COC) - Auto ignition temperature : N.A - Flammable limits in air(pct by volume) : N.A Upper : N.A Lower : N.A  Hazardous result after fire : It could be toxic carbon oxide.  Regulation of way to extinguish : The fire service act 2-4, Enforcement ordinance of the fire service act 12-1  Fire fighting : Move container from fire area if it can be done without risk. Do not scatter spilled material with high-pressure water streams. Dike for later disposal. Use extinguishing agents appropriate for surrounding fire. Avoid inhalation of material or combustion by –products. Stay upwind and keep out of low areas.

MATERIAL SAFETY DATA SHEET

6. SPILL OR LEAK PROCEDURE

■ Stop leak if possible without personal risk. Small spill : Absorb with sand or other non-combustible material. Collect spilled material in appropriate container for disposal. Keep unnecessary people away, isolate hazard area and deny entry.

7. HANDING AND STORAGE

■ Storage & Handling - Store and handle in accordance with all current regulations and standards. - Store cool place that air circulates well.

8. SPECIAL PROTECTION INFORMATION

 Respiratory protection : Handle in the presence of adequate ventilation. If the exposure limit is exceeded, a half-face organic vapor respirator may be worn. If it’s over 100℃, use the respirator supplier.  Eye protection : Wear splash resistant safety goggle. Provide and emergency eye wash fountain and quick drench shower in the immediate work area.  Other protective equipment : Wear adequate clothing to minimize physical contact  Protective gloves : Use chemical resistant gloves  Other instructions : N.A

9. PHYSICAL DATA

■ Appearance : Yellow pellet.  Odor : Slight smell of amine.  PH : 8.5 – 10.5(2% aq. sol, n)  Boiling point / range: N.A  Melting point / range : 45 - 55℃  Explosion : N.A  Oxidization : N.A ■ Vapor Pressure : N.A  Vapor density : N.A  Specific gravity : 0.874(60℃)  Solubility : Soluble in water.  Viscosity : 27cps(60℃)  Molecular weight : N.A  Distribution figures : N.A

10. STABILITY AND REACTIVITY

 Stability : Stable at normal temperature and pressure ■ Conditions to avoid : Avoid source of ignition, spark, heat. ■ Incompatibility (materials to be avoided) : Oxidant. ■ Hazardous decomposition products : Toxic carbon oxide could occur. ■ Hazardous polymerization : N.A

11. TOXICOLOGICAL INFORMATION

■ Toxicity data : N.A  Acute toxicity level : N.A  Possibility of causing cancer : N.A

12. ENVIROMENTAL HAZARDS

 Information of environmental poison : No.  Possibility to be accumulated in body : No  Is this poisonous to ecosystem / aquatic? : No  Poisonous to soil : No

13. DISPOSAL CONSIDERATION

 Regulation of the industrial waste control law : Industrial waste control law 25 ■ Waste disposal : All materials should be packaged, labeled, transported according to the local solid waste disposal regulation.

14. TRANSPORTATION INFORMATION

 Classification is not assigned

15. REGULATORY INFORMATION

 Industry safety law : No regulation  Toxic chemical material management law : Managed by the Method of preservation of water quality 2-2  The fire service act : No regulation

16. OTHER INFORMATION

 This is from OHS MSDS of MDI INFORMATION SYSTEM INC, The fire service act, Industrial waste control law.  Date prepared : Sep. 6. 1996  Time of edition / Revision date : 2 / Apr. 30. 1996.

The information contained herein is provided for the  purpose of disclosing product hazards and does not constitute product specifications.

All warranties, either expressed or implied, are hereby disclaimed.