1. IDENTIFICATION OF THE SUBSTANCE PREPARATION AND OF THE COMPANY

■ Product name : TOWREX ASMS 200 ■ Manufacturer : DONGBO S. C. CORPORATION 162-4, Sinduck-ri, Sungnam-Myoun Cheonan-si, Choongcheongnam-do, Korea Tel: +82-41-55 3-0883 / Fax: +82-41-553-0803 ■ Intended use : Plastic additive (Antistatic)

2. COMPOSITION / INFORMATION ON INGREDIENTS

Chemical Name; Quantity; FDA NO.; CAS NO.. Fatty amine ethoxilated; DFR21-178.3130; 26635-92-7. Polyol Ester; DFR21-184.1324; 31566-31-1.

3. HAZARDS IDENTIFICATION

 NFPA(level : 0-4) : Health=2, Fire=1, Reactivity=0. ---------------- Emergency Overview---------------------------------- This could irritate respiratory organ and might cause burned skin and eyes.. ---------------- Potential health effects ------------------------------:  Eye contact: - Long term exposure : Might get burned.  Skin contact: - Long term exposure : Might get burned.  Inhalation : - Short term exposure :May cause slight irritation.  Ingestion : - Long term exposure : Might get burned

4. FIRST AID MEASURES

■ Inhalation : Remove from exposure immediately. Be medically aided if symptoms such as cough become worse.. ■ Skin : Remove everything you were wearing such as contaminated clothing , shoes, accessories, then should wash them(about 15 – 20 min)with detergent and large amounts of water.. Contact a physician if irritation continues ■ Eye : Immediately wash eyes with large amount of water or normal saline for 15 minutes, occasionally lifting upper and lower lids, until no evidence of chemical remains, Get medical attention. ■ Ingestion : Induce to throw up and contact a physician.

5. FREE-FIGHTING MEASURES

■ Extinguish media : Regular dry chemical, carbon dioxide, water and foam.  Fire And explosion hazards - Flash point : >200℃ - Auto ignition temperature : N.A - Flammable limits in air(pct by volume) : N.A Upper : N.A Lower : N.A  Special fire fighting procedures : - Move the container from fire area if there is no danger.. - Use water spray to cool fire exposed containers.. - Reduce smoke using water spray. - Avoid breathing smoke, fumes and decomposition products  Hazardous result after fire : Carbon oxide, nitrogen oxide.

MATERIAL SAFETY DATA SHEET

6. SPILL OR LEAK PROCEDURE

■ Steps to be taken in case of release or spill : - Transfer spilled material to secure containers. Do not allow entering into drains or watercourse. - For health : Isolate hazard area, then shouldn’t take in dust and smoke using mask or something - All materials collected should be thrown away according to the local solid waste disposal regulation.

7. HANDING AND STORAGE

■ Storage & Handling - Store and handle in accordance with all current regulations and standards. - Avoid air or light. Keep it in airtight container.

8. SPECIAL PROTECTION INFORMATION

 Respiratory protection : Handle in the presence of adequate ventilation. Wear a dust mask if dust is present.  Eye protection : Wear splash resistant safety goggle. Provide and emergency eye wash fountain and quick drench shower in the immediate work area.  Other protective equipment : Wear adequate clothing to minimize physical contact  Protective gloves : Use chemical resistant gloves  Other instructions : N.A

9. PHYSICAL DATA

■ Appearance : Slightly yellow pellet.  Odor : slight smell of amine  PH(1% sol,n) : 6 - 8 ■ Melting point : 45~52℃  Boiling point : N.A ■ Vapor Pressure : N.A  Vapor density : N.A  Solubility : Aromatic solvent, Chloroform  Viscosity : 30cps(100℃), 42cps(80℃)  Specific Gravity : 0.886(80℃), 0.869(100℃)

10. STABILITY AND REACTIVITY

 Stability : Stable at normal temperature and pressure ■ Conditions to avoid : Avoid source of ignition, spark, heat. ■ Incompatibility (materials to be avoided) : Oxidant, reducing agent. ■ Hazardous decomposition products : Carbon oxide, nitrogen oxide.. ■ Hazardous polymerization : N.A

11. TOXICOLOGICAL INFORMATION

■ Toxicity data - Fatty amine : LD50(Akso) -> 1850 mg/kg(rat, oral) - Polyol ester : LD50 -> 5 mg/kg(rat, oral)  Possibility of causing cancer : N.A

12. ENVIROMENTAL HAZARDS

 Information of environmental poison : No  Possibility to be accumulated in body : No  Is this poisonous to ecosystem / aquatic? : No  Poisonous to soil : No

13. DISPOSAL CONSIDERATION

■ Waste disposal : All materials should be packaged, labeled, transported according to the local solid waste disposal regulation.

14. TRANSPORTATION INFORMATION

 Classification is not assigned

15. REGULATORY INFORMATION

 Industry safety law : No regulation  Toxic chemical material management law : No regulation  The fire service act : No regulation

16. OTHER INFORMATION

This information is from MSDS of Korea Occupational Safety & Healthy Agency.

The information contained herein is provided for the  purpose of disclosing product hazards and does not constitute product specifications.

All warranties, either expressed or implied, are hereby disclaimed.