1. IDENTIFICATION OF THE SUBSTANCE PREPARATION AND OF THE COMPANY

■ Product name : TOWREX NU500 ■ Manufacturer : DONGBO S. C. CORPORATION 162-4, Sinduck-ri, Sungnam-Myoun Cheonan-si, Choongcheongnam-do, Korea Tel: +82-41-55 3-0883 / Fax: +82-41-553-0803 ■ Intended use : Plastic additive (Nucleating agent)

2. COMPOSITION / INFORMATION ON INGREDIENTS

Number; % w/w; CAS No.; Chemical name. 1; 100; 54686-97-4; Bis (p-methyl benzylidene) Sorbitol.

3. PHYSIOLOGICAL AND HEALTH HAZARD DATA

■ Effects of overexposure : - Eyes : Dust may cause slight irritation. Permanent injury is not known. - Skin : It may cause slight irritation. - Inhalation : Dust excessive amount may cause irritation of respiratory organs. - Ingestion : Fatal injury is not known.

4. FIRST AID MEASURES

■ General In all cases of doubt, when symptoms persist, seek medical attention. ■ Inhalation : Move to the place with fresh air. Be medically aided if symptoms such as cough become worse. ■ Skin : Immediately wash with large amounts of soap and water. Remove and wash contaminated clothing before reuse. Contact a physician if irritation continues. ■ Eye : Immediately flush eyes with enough water for 15 minutes. ■ Ingestion : Induce to throw up and contact a physician.

5. FREE-FIGHTING MEASURES

■ Extinguish media : Water, carbon dioxide, foam or dry powder ■ Fire and explosion hazards - Flash point (COC) : > 250℃ - Auto ignition temperature : N.A - Flammable limits in air (pct by volume) Upper : N.A Lower : N.A ■ Special fire fighting procedures : - Use water spray to cool fire exposed containers. - Evacuate non-emergency personal to a safe area. - Avoid breathing smoke, fumes and decomposition products. - Wear self-contained breathing apparatus. ■ Unusual fire or explosion hazards : - Dust may form flammable or explosive mixtures when dispersed in air in high dust concentrations

MATERIAL SAFETY DATA SHEET

6. SPILL OR LEAK PROCEDURE

■ Steps to be taken in case of release or spill : - Wear appropriate protective equipment as specified in section 8 - Transfer spilled material to secure containers. ■ Waste disposal method : All materials should be packaged, labeled, transported according to the local solid waste disposal regulation.

7. HANDKING AND STORAGE

■ Handling - Avoid contact with skin and eyes - When using do not eat, drink or smoke ■ Storage - Store at ambient temperature - Store in a dry place

8. SPECIAL PROTECTION INFORMATION

■ Respiratory protection : - Handle in the presence of adequate ventilation. - Wear a dust mask if dust is present. ■ Protective gloves : Use chemical resistant gloves. ■ Eye protection : Use chemical splash goggles. ■ Other protective equipment : Adequate clothing to minimize physical contact. ■ Other instructions : N.A

9. PHYSICAL DATA

■ Appearance : White powder ■ Melting point : 245~255 ℃ ■ Vapor Pressure(mmHg) : N.A ■ Water solubility : Insoluble ■ Pct volatiles by weight at 100℃ : < 1.0% ■ Odor : Slightly sweet

10. STABILITY AND REACTIVITY

■ Stability : Stable ■ Conditions to avoid : pH < 5 , pH > 9 ■ Incompatibility (materials to be avoided) : N.A ■ Hazardous decomposition products : - Decomposition may produce fumes, smokes, oxides, of carbon and/or hydrocarbon. ■ Hazardous polymerization : will not occur. ■ Conditions to avoid : N.A

11. TOXICOLOGICAL INFORMATION

■ Threshold limit value : Not established. ■ Median lethal dosage : LD50 (Rats, Oral) - 12 g/kg

12. ENVIROMENTAL HAZARDS

N. A

13. DISPOSAL CONSIDERATION

■ Waste disposal : All materials should be packaged, labeled, transported according to the local solid waste disposal regulation.

14. TRANSPORTATION INFORMATION

■ Department of transportation classification □ FLAMMABLE LIQUID □ FLAMMABLE SOLID □ FLAMMABLE GAS □ COMBUSTIBLE LIQUID □ OTHER SPECIFY BELOW; □ POISON, CLASS A □ POISON, CLASS B □ OXIDIZING MATERIAL □ CORROSIVE MATERIAL; □ IRRITATING MATERIAL □ NON FLAMMABLE GAS ▣ NOT HAZARDOUS BY D.O.T REGULATIONS. ■ D.O.T. Proper shipping name : N. A

15. REGULATORY INFORMATION

This material and / or its components are listed under the TSCA

16. OTHER INFORMATION

Date prepared: Mar. 20. 1995. Revision date : Oct. 28. 2003 Approved by : K. J. KIM / Environmental manager

The information contained herein is provided for the  purpose of disclosing product hazards and

does not constitute product specifications.

All warranties, either expressed or implied, are hereby disclaimed.