1. IDENTIFICATION OF THE SUBSTANCE PREPARATION AND OF THE COMPANY

■ Product name : TOWREX NU 500P ■ Manufacturer : DONGBO S. C. CORPORATION 162-4, Sinduck-ri, Sungnam-Myoun Cheonan-si, Choongcheongnam-do, Korea Tel: +82-41-55 3-0883 / Fax: +82-41-553-0803 ■ Intended use : Plastic additive (Nucleating agent)

2. COMPOSITION / INFORMATION ON INGREDIENTS

Number; % w/w; CAS No.; Chemical name. 1; 100; 93379-37-4; Hexitol, 1,3:2,4-bis-O-[(4-methylphenyl)methylene]-.

3. PHYSIOLOGICAL AND HEALTH HAZARD DATA

■ Effects of overexposure : - Eyes : Dust may cause slight irritation. Permanent injury is not known. - Skin : It may cause slight irritation. - Inhalation : Dust excessive amount may cause irritation of respiratory organs. - Ingestion : Fatal injury is not known.

4. FIRST AID MEASURES

■ General In all cases of doubt, when symptoms persist, seek medical attention. ■ Inhalation : Move to the place with fresh air. Be medically aided if symptoms such as cough become worse. ■ Skin : Immediately wash with large amounts of soap and water. Remove and wash contaminated clothing before reuse. Contact a physician if irritation continues. ■ Eye : Immediately flush eyes with enough water for 15 minutes. ■ Ingestion : Induce to throw up and contact a physician.

5. FREE-FIGHTING MEASURES

■ Extinguish media : Water, carbon dioxide, foam or dry powder ■ Fire and explosion hazards - Flash point (COC) : > 250℃ - Auto ignition temperature : N.A - Flammable limits in air (pct by volume) Upper : N.A Lower : N.A ■ Special fire fighting procedures : - Use water spray to cool fire exposed containers. - Evacuate non-emergency personal to a safe area. - Avoid breathing smoke, fumes and decomposition products. - Wear self-contained breathing apparatus. ■ Unusual fire or explosion hazards : - Dust may form flammable or explosive mixtures when dispersed in air in high dust concentrations

MATERIAL SAFETY DATA SHEET

6. SPILL OR LEAK PROCEDURE

■ Steps to be taken in case of release or spill : - Wear appropriate protective equipment as specified in section 8 - Transfer spilled material to secure containers. ■ Waste disposal method : All materials should be packaged, labeled, transported according to the local solid waste disposal regulation.

7. HANDKING AND STORAGE

■ Handling - Avoid contact with skin and eyes - When using do not eat, drink or smoke ■ Storage - Store at ambient temperature - Store in a dry place

8. SPECIAL PROTECTION INFORMATION

■ Respiratory protection : - Handle in the presence of adequate ventilation. - Wear a dust mask if dust is present. ■ Protective gloves : Use chemical resistant gloves. ■ Eye protection : Use chemical splash goggles. ■ Other protective equipment : Adequate clothing to minimize physical contact. ■ Other instructions : N.A

9. PHYSICAL DATA

■ Appearance : White powder ■ Melting point : 235~255 ℃ ■ Vapor Pressure(mmHg) : N.A ■ Water solubility : Insoluble ■ Pct volatiles by weight at 100℃ : < 1.0% ■ Odor : Slightly sweet

10. STABILITY AND REACTIVITY

■ Stability : Stable ■ Conditions to avoid : pH < 5 , pH > 9 ■ Incompatibility (materials to be avoided) : N.A ■ Hazardous decomposition products : - Decomposition may produce fumes, smokes, oxides, of carbon and/or hydrocarbon. ■ Hazardous polymerization : will not occur. ■ Conditions to avoid : N.A

11. TOXICOLOGICAL INFORMATION

■ Threshold limit value : Not established. ■ Median lethal dosage : LD50 (Rats, Oral) - 12 g/kg

12. ENVIROMENTAL HAZARDS

N. A

13. DISPOSAL CONSIDERATION

■ Waste disposal : All materials should be packaged, labeled, transported according to the local solid waste disposal regulation.

14. TRANSPORTATION INFORMATION

■ Department of transportation classification □ FLAMMABLE LIQUID □ FLAMMABLE SOLID □ FLAMMABLE GAS □ COMBUSTIBLE LIQUID □ OTHER SPECIFY BELOW; □ POISON, CLASS A □ POISON, CLASS B □ OXIDIZING MATERIAL □ CORROSIVE MATERIAL; □ IRRITATING MATERIAL □ NON FLAMMABLE GAS ▣ NOT HAZARDOUS BY D.O.T REGULATIONS. ■ D.O.T. Proper shipping name : N. A

15. REGULATORY INFORMATION

This material and / or its components are listed under the TSCA

16. OTHER INFORMATION

This Information is from MSDS of Korea Occupational Safety & Healthy Agency. Date prepared: Oct. 05. 1996. Revision date : July. 23. 2008 Approved by : J. I. RYU / Environmental manager The The information contained herein is provided for the purpose of disclosing product hazards and does not constitute product specifications. All warranties, either expressed or implied, are hereby disclaimed.